	Changing the Brushes-Holder on Power Packs

	Dismantle
a)
Put one mark on the casing item 1 and one on the flange item 2 (to be able to reassemble the unit in the same position).

b)
Remove nuts item 3 and washers item 4.

c)
Remove the flange item 2.

d)
Remove the fasteners item 4 and the connector item 6 by pushing the catch item 8 located on the fasteners. Mark the location of the red wire on connector item 6.

e)
Remove the brushes-holder by unscrewing screws item 7.

Reassemble

f)
Replace in position the brushes-holder and tighten screws item 7.

g)
Slightly reopen the catches item 8 and position the fasteners item 4. Make sure that the red cable is correctly located.

h)
Assemble the flange item 2 and the casing item 1 by aligning the two markings mentioned in paragraph n° a).

i)
Push apart the two brushes with a small screwdriver in order to place them on the shaft item 9.

j) Carry on the assembly work and push apart the two brushes in order to place them on the collector item 10.

Check the two marks for correct alignment. Put back the two washers item 4 and tighten the two bolts item 3.

	[image: image1.jpg]





	SA LECOMBLE & SCHMITT – B.P. n°2 – 64240 URT – France

	Téléphone 33(0) 5.59.56.22.22 – Télécopie 33 (0) 5.59.56.27.33

	E-mail commercial2@ls-france.com


